Wet Retention Basin

Operation and Maintenance Agreement

I will keep a maintenance record on this BMP.  This maintenance record will be kept in a log in a known set location.  Any deficient BMP elements noted in the inspection will be corrected, repaired or replaced immediately.  These deficiencies can affect the integrity of structures, safety of the public, and removal efficiency of the BMP.

The wet detention basin system is defined as the wet detention basin, pretreatment including forebays and the vegetated filter if one is provided.

This system (check one):

Does        	does not              incorporate a vegetated filter at the outlet.

This system (check one):

Does	    	does not	        incorporate pretreatment other than a forebay.

Important maintenance procedures:

- Immediately after the wet detention basin is established, the plants on the vegetated shelf and perimeter of the basin should be watered twice weekly if needed, until the plants become established (commonly six weeks).

- No portion of the wet detention pond should be fertilized after the first initial fertilization that is required to establish the plants on the vegetated shelf.

- Stable groundcover should be maintained in the drainage area to reduce the sediment load to the wet detention basin.

- If the basin must be drained for an emergency or to perform maintenance, the flushing of sediment through the emergency drain should be minimized to the maximum extent practical.

After the wet extended detention basin is established, it will be inspected once a month and within 24 hours after every storm event greater than 1.0 inches.  Records of operation and maintenance will be kept in a known set location and will be available upon request.

Inspection activities shall be performed as follows.  Any problems that are found shall be repaired immediately.

| BMP element: | Potential problem: | How I will remediate the problem: |
| --- | --- | --- |
| The entire BMP | Trash/debris is present. | Remove the trash / debris. |
| The perimeter of the wet detention basin | Areas of bare soil and/or erosive gullies have formed. | Re-grade the soil if necessary to remove the gully, and then plant a ground cover and water until it is established. Provide lime and a one-time fertilizer application. |
|  | Vegetation is too short or too long. | Maintain vegetation at a height of approximately six inches. |
| The inlet device: pipe or swale | The pipe is clogged. | Unclog the pipe. Dispose of the sediment off-site. |
|  | The pipe is cracked or otherwise damaged. | Replace the pipe. |
|  | Erosion is occurring in the swale. | Re-grade the swale if necessary to smooth it over and provide erosion control devices such as reinforced turf matting or riprap to avoid future problems with erosion. |
| The forebay | Sediment has accumulated to a depth greater than the original design depth for sediment storage. | Search for the source of the sediment and remedy the problem if possible. Remove the sediment and dispose of it in a location where it will not cause impacts to streams or the BMP. |
|  | Erosion has occurred. | Provide additional erosion protection such as reinforced turf matting or riprap if needed to prevent future erosion problems. |
|  | Weeds are present. | Remove the weeds, preferably by hand. If pesticide is used, wipe it on the plants rather than spraying. |
| The vegetated shelf | Best professional practices show that pruning is needed to maintain optimal plant health. | Prune according to best professional practices. |
| The vegetated shelf (cont.) | Plants are dead, diseased or dying. | Determine the source of the problem: soils, hydrology, disease, etc. Remedy the problem and replace plants. Provide one-time fertilizer application to establish the ground cover if a soil test indicates it is necessary. |
|  | Weeds are present. | Remove the weeds, preferably by hand. If pesticide is used, wipe it on the plants rather than spraying. |
| The main treatment area | Sediment has accumulated to a depth greater than the original design sediment storage depth. | Search for the source of the sediment and remedy the problem if possible. Remove the sediment and dispose of it in a location where it will not cause impacts to streams or the BMP. |
|  | Algal growth covers over 50% of the area. | Consult a professional to remove and control the algal growth. |
|  | Cattails, phragmites or other invasive plants cover 50% of the basin surface. | Remove the plants by wiping them with pesticide (do not spray). |
| The embankment | Shrubs or trees have started to grow on the embankment. | Remove shrubs or trees immediately, and restore embankment to as built standards. |
|  | Evidence of muskrat or beaver activity is present. | Use traps to remove muskrats and consult a professional to remove beavers. |
|  | An annual inspection, by an appropriate professional, shows that the embankment needs repair (if applicable). | Make all needed repairs. |
| The outlet device | Clogging has occurred. | Clean the outlet device. Dispose of the sediment off-site. |
|  | The outlet device is damaged. | Repair or replace the outlet device. |
| The receiving water | Erosion or other signs of damage have occurred at the outlet. | Contact the City of Kings Mountain Stormwater Department at 704-734-4501 or stormwater@cityofkm.com. |

The measuring device used to determine the sediment elevation shall be such that it will give an accurate depth reading and not readily penetrate into accumulated sediments.

When the permanent pool depth reads______ feet in the main pond, the sediment shall be removed.

When the permanent pool depth reads______ feet in the forebay, the sediment shall be removed.

I acknowledge and agree by my signature below that I am responsible for the performance of the maintenance procedures listed above.  I agree to notify The City of Kings Mountain Stormwater Department of any problems with the system or prior to any changes to the system or responsible party.

Project name:__________________________________________________________________

BMP type:_____________________________________________________________________

Print name:__________________________________________________________

Title:________________________________________________________________

Address:_____________________________________________________________

Phone;______________________________________________________________

Signature:____________________________________________________________

Date:________________________________________________________________

Note: the legally responsible party should not be a homeowners association unless more than 50% of the lots have been sold and a resident of the subdivision has been named president.

I, ____________________________________, a Notary Public for the State of ___________________________, County of ______________________, do hereby certify that ______________________________personally appeared before me this ________________ day of _________________, _______, and acknowledged the due execution of the forgoing wet detention basin maintenance requirements.  Witness my hand and official seal,

SEAL

My commission expires _____________________________